# Embedded Programming with Microcontrollers

# Exploring the Circuit Playground Express

# Overview

The Circuit Playground Express is an example of microcontroller based electronics.  It contains a number of input/output contacts, neopixels, and sensors that can be programmed with various programming environments:  Microsoft Makecode, Javascript, Python, and the Arduino development environment.

# CircuitPython

CircuitPython is the implementation of Python created by Adafruit for several products, including the Circuit Playground Express.  It is a fork/derivative of MicroPython, a version of Python designed to run on microcontrollers.  CircuitPython excels at the following:

- Very fast development: Write code, save the file, and it runs immediately.  No compiling is required, since Python is an interpreted language – the hardware can interpret and act on each command you type, practically simultaneously.

- REPL – Read Evaluate Print Loop: You can start interactive programming with the REPL

- Easy code changes: Since your code lives on the flash drive, you can edit it whenever you like, and you can keep multiple files around for easy experimentation.

- File Storage: CircuitPython’s data storage ability makes it great for data logging, playing audio clips, and otherwise interacting with files.

# Using the Mu Editor

Install the Mu Editor from here:  https://codewith.mu/

- Before starting Mu, please plugin your Circuit Playground Express and make sure it shows up as a CIRCUITPY drive in your computer’s file explorer/finder.  When you are finished, you will eject this drive safely, just as you would any flash drive.

- Start Mu, and select the Adafruit CircuitPython mode.

- Managing files:  The CircuitPython file you copy over to run on the Circuit Playground Express should always be called code.py.  This is the file the device is looking to run.  When you are finished creating, testing, and running this file, you should create another copy of this file on your own machine with a more appropriate filename; e.g. blinkingLED.py

# Creating our First Program

Our first program is made of the following code:

import time

from adafruit_circuitplayground.express import cpx

while True:

cpx.red_led = True

time.sleep(1)

cpx.red_led = False

time.sleep(1)

## Running our First Program

Copy a file named code.py onto the CIRCUITPY drive.  You can use the Save button to save the file directly to the Circuit Playground Express if you named the program code.py.  If not, double-click the filename tab, and a dialog box will appear allowing you to name and save the code wherever you wish.

The program should automatically start blinking the Pin 13 LED (beside the Micro USB jack) repeatedly in 1 second intervals.  If the LED is not blinking, press the Reset button.

## Serial Output

Insert the line:   print(“Hello CircuitPython!”) into the while loop.  Copy the new version of the code.py file to the Circuit Playground Express and the Serial Window will open in the Mu editor.  The Serial window acts a both a command input/output window and as an interactive REPL Python environment.

# Next Steps

## Using the NeoPixels

The NeoPixel LEDs are numbered counterclockwise from cpx.pixels[0] through cpx.pixels[9].

All pixels can be lit to the same colour value specified by the RGB colour scheme.  For example:

cpx.pixels.fill( 30, 0, 0)

will set all pixels to a red colour.  The full ON value is 255 and the full OFF value is 0.  Setting the (R, G, B) values in different ways produces different colours.  The following program sets the NeoPixel LEDs to a series of random colours, every 5 seconds.

import time

import random

from adafruit_circuitplayground.express import cpx

cpx.pixels.brightness = 0.1

while True:

for i in range(10):

r = random.randint(0,255)

g = random.randint(0,255)

b = random.randint(0,255)

cpx.pixels[i] = (r,g,b)

time.sleep(5)

## Reading Sensors

import time

from adafruit_circuitplayground.express import cpx

while True:

print(“Temperature: “, cpx.temperature, “, Light Intensity: “, cpx.light)

time.sleep(1)

The following program shows how to get things started when it comes to reading sensors.

## Mu Plotting

import time

from adafruit_circuitplayground.express import cpx

while True:

intensity = cpx.light

print( (intensity, ) )

time.sleep(0.1)

Mu has a built-in plotting capability that allows you to visualize what a sensor is doing as data is being collected.  Outputting a Tuple using the print() will provide a real-time plot of each item in the Tuple relative to time.  The Plotter icon must be clicked in order to see the plotted values.  The following program shows how to plot the light intensity values read by the light sensor.

## Challenges

- Create a program that: turns all NeoPixels green, waits 5 seconds, turns all NeoPixels yellow, waits two seconds, then turns all NeoPixels red for 5 seconds.  This sequence should repeat indefinitely.

- Create a program that rotates a single red-light counter-clockwise from through NeoPixels 0 to 9.  This sequence should repeat indefinitely.

- Create a program that rotates a single red-light counter-clockwise from through NeoPixels 0 to 9 and then clock-wise through NeoPixels 9 to 0.  This sequence should repeat indefinitely.

# Exploring the ItsyBitsy M4 Express

# Overview

The ItsyBitsy M4 Express is a small microcontroller board that can be programming using either the Arduino IDE or CircuitPython.  With CircuitPython you can use the Mu editor, if you’d like, or any text editor of your choosing.  You actually don’t need to install any IDE onto your computer.  Just edit the main.py file on the board (it shows up like a flash drive), save it, and your Python code will run.  Note: Loose headers come with the board and must be soldered on to be used with a breadboard.

## Examples Using a Breadboard:

import board

import digitalio

import time

led = digitalio.DigitalInOut(board.D13)

led.direction = digitalio.Direction.OUTPUT

while True:

led.value = True

time.sleep(0.5)

led.value = False

time.sleep(0.5)

Built-in Blinking LED

### Digital Input/Output

import time

import board

from digitalio import DigitalInOut, Direction, Pull

led = DigitalInOut(board.D13)

led.direction = Direction.OUTPUT

switch = DigitalInOut(board.D2)

switch.direction = Direction.INPUT

switch.pull = Pull.UP

while True:

if switch.value:

led.value = False

else:

led.value = True

time.sleep(0.01)  # debounce delay

import time

import board

import pulseio

piezo = pulseio.PWMOut(board.A1, duty_cycle=0, frequency=440, variable_frequency=True)

while True:

for f in (262, 294, 330, 349, 392, 440, 494, 523):

piezo.frequency = f

piezo.duty_cycle = 65536 // 2  # On 50%

time.sleep(0.25)  # On for 1/4 second

piezo.duty_cycle = 0  # Off

time.sleep(0.05)  # Pause between notes

time.sleep(0.5)

Pulse Width Modulation

## Challenge

- Given:  a red LED, a 220 Ohm resistor, and 2 jumper wires.  Create a circuit and program that: blinks the red LED once every second.  Note:  The circuit can be created between the ground (G) pin and pin 7. Remember:  The long lead on the red LED is the positive one.

## Resources

What is Circuit Python? - https://learn.adafruit.com/welcome-to-circuitpython/what-is-circuitpython

Circuit Python Libraries - https://learn.adafruit.com/welcome-to-circuitpython/circuitpython-libraries

Circuit Python Essentials - https://learn.adafruit.com/welcome-to-circuitpython/circuitpython-essentials

Circuit Python - https://circuitpython.org/

Circuit Playground Express Library - https://circuitpython.readthedocs.io/projects/circuitplayground/en/latest/api.html

Introducing Adafruit ItsyBitsy M4 Express - https://learn.adafruit.com/introducing-adafruit-itsybitsy-m4

## Where to Buy?

Mouser Canada - https://www.mouser.ca/

Elmwood Electronics - https://elmwoodelectronics.ca/